Port Layout and Parameters

Survey Hazira Port berth inventory: 11 berths (3 container, 5 general cargo, 3 liquid), channel draft 11 m, yard footprints (open: 80 000 m2; container yard: 50 ha), and assumed handling rates (25000t/day bulk; 30 moves/hr crane).

Hazira Port (Wikipedia)

- Also known as Surat Port

- Deep-water liquefied natural gas terminal

  - Liquefied natural gas terminals are used for managing import and export of LNG (which is a natural gas that is cooled to a liquid - source)

  - Deep-water meaning that it is a manmade structure located beyond state boundaries in the water (source)

  - Designed for Essar Steel – now owned by Adani Group

  - Located 20 miles southwest of Surat

Berths: “a designated location in a port or harbor used for mooring vessels,” berths are numbered and matched to ships based on length, draft (the vertical distance between the bottom of the ship and the waterline - source) and cargo type (source)

- MP1-MP4: multipurpose berths for liquid/bulk cargo

- CT1, CT2: container berths

- Berthing speed limit: 10cm/sec

- MP1-MP3, CT1, CT2 has maximum permissible draft of 14m, MP4 has 13.0

Channel: also known as an approach channel, is a waterway for vessels to navigate between the water and berths (source)

- Water depth in approach channel: 15.5m

- Minimum UKC in approach channel is 15% of maximum draft and 10% of maximum draft in basement

- Minimum UKC alongside berth: 1m

  - UKC = “under keel clearance”

- “Maximum current in the approach channel at one ship’s length outside the breakwaters is 2.5 knots. Cross – current in opposite direction in the channel are not acceptable” (p. 8)

- “Minimum UKC not below 15% of the deepest draft in the channel, not below 10% of the deepest draft in the turning circle and inner basin and not below 1.0 meter at the berth.” (p. 8)

- “The limiting wave height is Hs = 1.5 m in combination with a wind speed of 24 Knots (10 minutes average wind speed).” (p. 8)

Yard Footprints

- “Currently we have 32 acres for empty container storage of 12000 TEU with an expansion capacity of another 10 acres” (source)

p. 51 - General Layout

Handling Rates (p. 9)

Average Handling Rates (p. 63)

- “The number of berths required, is a function of cargo type and volume, and the expected cargo handling rates” (p. 63)

Sources for Research on Hazira Port:

- Adani Ports official document

- Adani - Berthing Policy

- Shell - Port Information Book

- Detailed map

Simulation Resources

- Paper on Complex Port Simulations